#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include <string.h>

#include <stdio.h>

#include "lcd.h" // فرض بر این است که کتابخانه ای برای LCD وجود دارد

#define F_CPU 8000000UL

// ADC Channels

#define LM35_PIN 0

#define LDR_PIN 1

// Motor Control Pins (PWM)

#define MOTOR1_PWM_PIN  PB1 // OC1A

#define MOTOR2_PWM_PIN  PB2 // OC1B

// LED Pins

#define LED_VERY_COLD  PD0

#define LED_COLD      PD1

#define LED_TEMPERATE PD2

#define LED_WARM      PD3

#define LED_TOO_WARM  PD4

#define LED_LIGHT     PD5

#define LED_DARK      PD6

// Button Pins

#define BTN_ENTER  PC0

#define BTN_BACK   PC1

#define BTN_RIGHT  PC2

#define BTN_LEFT   PC3

// Temperature thresholds

int LL = 10, L = 30, H = 60, HH = 80;

// Password and user states

const char* username = "111";

const char* password = "12345";

char entered_password[6] = "";

int password_index = 0;

// State variables

int current_temp = 0;

int current_ldr = 0;

// Time tracking variables

volatile uint32_t seconds = 0;

uint32_t mode_durations[5] = {0};

// Menu states

enum MenuState { MAIN_MENU, SET_LL, SET_L, SET_H, SET_HH, PASSWORD_MENU, USER_PAGE, SETTINGS_MENU } menuState = MAIN_MENU;

int menuIndex = 0;

int subMenuIndex = 0;

int user_logged_in = 0;

// Timer flag

volatile uint8_t timer_flag = 0;

void ADC_init() {

ADMUX = (1<<REFS0); // AVcc with external capacitor at AREF pin

ADCSRA = (1<<ADEN) | (1<<ADPS2) | (1<<ADPS1) | (1<<ADPS0); // Enable ADC and set prescaler to 128

}

uint16_t ADC_read(uint8_t ch) {

ch &= 0b00000111;  // Mask channel number to be between 0 and 7

ADMUX = (ADMUX & 0xF8) | ch; // Clear the older channel that was read, and set the new one

ADCSRA |= (1<<ADSC); // Start conversion

while(ADCSRA & (1<<ADSC)); // Wait for conversion to complete

return ADC;

}

void LCD_init() {

lcd_init(LCD_DISP_ON);

lcd_clrscr();

}

void update_LCD() {

char buffer[20];

lcd_clrscr();

switch(menuState) {

case MAIN_MENU:

lcd_gotoxy(0, 0);

lcd_puts("Main Menu:");

lcd_gotoxy(0, 1);

if (menuIndex == 0) lcd_puts("> Set LL");

else lcd_puts("  Set LL");

lcd_gotoxy(0, 2);

if (menuIndex == 1) lcd_puts("> Set L");

else lcd_puts("  Set L");

lcd_gotoxy(0, 3);

if (menuIndex == 2) lcd_puts("> Set H");

else lcd_puts("  Set H");

lcd_gotoxy(0, 4);

if (menuIndex == 3) lcd_puts("> Set HH");

else lcd_puts("  Set HH");

lcd_gotoxy(0, 5);

if (menuIndex == 4) lcd_puts("> User Page");

else lcd_puts("  User Page");

break;

case SET_LL:

lcd_gotoxy(0, 0);

lcd_puts("Set LL:");

lcd_gotoxy(0, 1);

sprintf(buffer, "%d", LL);

lcd_puts(buffer);

break;

case SET_L:

lcd_gotoxy(0, 0);

lcd_puts("Set L:");

lcd_gotoxy(0, 1);

sprintf(buffer, "%d", L);

lcd_puts(buffer);

break;

case SET_H:

lcd_gotoxy(0, 0);

lcd_puts("Set H:");

lcd_gotoxy(0, 1);

sprintf(buffer, "%d", H);

lcd_puts(buffer);

break;

case SET_HH:

lcd_gotoxy(0, 0);

lcd_puts("Set HH:");

lcd_gotoxy(0, 1);

sprintf(buffer, "%d", HH);

lcd_puts(buffer);

break;

case PASSWORD_MENU:

lcd_gotoxy(0, 0);

lcd_puts("Enter Password:");

lcd_gotoxy(0, 1);

lcd_puts(entered_password);

break;

case USER_PAGE:

lcd_gotoxy(0, 0);

lcd_puts("User Page:");

lcd_gotoxy(0, 1);

lcd_puts("Temp: ");

sprintf(buffer, "%d", current_temp);

lcd_puts(buffer);

lcd_gotoxy(0, 2);

lcd_puts("LDR: ");

sprintf(buffer, "%d", current_ldr);

lcd_puts(buffer);

break;

case SETTINGS_MENU:

lcd_gotoxy(0, 0);

lcd_puts("Settings:");

lcd_gotoxy(0, 1);

if (subMenuIndex == 0) lcd_puts("> View State 1");

else lcd_puts("  View State 1");

lcd_gotoxy(0, 2);

if (subMenuIndex == 1) lcd_puts("> View State 2");

else lcd_puts("  View State 2");

lcd_gotoxy(0, 3);

if (subMenuIndex == 2) lcd_puts("> View State 3");

else lcd_puts("  View State 3");

lcd_gotoxy(0, 4);

if (subMenuIndex == 3) lcd_puts("> View State 4");

else lcd_puts("  View State 4");

lcd_gotoxy(0, 5);

if (subMenuIndex == 4) lcd_puts("> View State 5");

else lcd_puts("  View State 5");

lcd_gotoxy(0, 6);

if (subMenuIndex == 5) lcd_puts("> Change LL");

else lcd_puts("  Change LL");

lcd_gotoxy(0, 7);

if (subMenuIndex == 6) lcd_puts("> Change L");

else lcd_puts("  Change L");

lcd_gotoxy(0, 8);

if (subMenuIndex == 7) lcd_puts("> Change HH");

else lcd_puts("  Change HH");

lcd_gotoxy(0, 9);

if (subMenuIndex == 8) lcd_puts("> Change H");

else lcd_puts("  Change H");

lcd_gotoxy(0, 10);

if (subMenuIndex == 9) lcd_puts("> Change Password");

else lcd_puts("  Change Password");

lcd_gotoxy(0, 11);

if (subMenuIndex == 10) lcd_puts("> Restore Settings");

else lcd_puts("  Restore Settings");

lcd_gotoxy(0, 12);

if (subMenuIndex == 11) lcd_puts("> Logout");

else lcd_puts("  Logout");

break;

}

}

void control_motors() {

if (current_temp <= LL) {

OCR1A = 255;  // Heater at max speed

OCR1B = 0;    // Cooler off

} else if (current_temp <= L) {

OCR1A = 155;  // Heater at min speed

OCR1B = 0;    // Cooler off

} else if (current_temp < H) {

OCR1A = 0;    // Heater off

OCR1B = 0;    // Cooler off

} else if (current_temp < HH) {

OCR1A = 0;    // Heater off

OCR1B = 155;  // Cooler at min speed

} else {

OCR1A = 0;    // Heater off

OCR1B = 255;  // Cooler at max speed

}

}

void update_LEDs() {

PORTD &= ~((1<<LED_VERY_COLD) | (1<<LED_COLD) | (1<<LED_TEMPERATE) | (1<<LED_WARM) | (1<<LED_TOO_WARM));

if (current_temp <= LL) {

PORTD |= (1<<LED_VERY_COLD);

} else if (current_temp <= L) {

PORTD |= (1<<LED_COLD);

} else if (current_temp < H) {

PORTD |= (1<<LED_TEMPERATE);

} else if (current_temp < HH) {

PORTD |= (1<<LED_WARM);

} else {

PORTD |= (1<<LED_TOO_WARM);

}

if (current_ldr < 50) {

PORTD |= (1<<LED_DARK);

} else {

PORTD |= (1<<LED_LIGHT);

}

}

void handle_buttons() {

if (bit_is_clear(PINC, BTN_ENTER)) {

_delay_ms(50); // Debounce

if (bit_is_clear(PINC, BTN_ENTER)) {

if (menuState == MAIN_MENU) {

switch (menuIndex) {

case 0: menuState = SET_LL; break;

case 1: menuState = SET_L; break;

case 2: menuState = SET_H; break;

case 3: menuState = SET_HH; break;

case 4: menuState = PASSWORD_MENU; break;

}

} else if (menuState == PASSWORD_MENU) {

if (strcmp(entered_password, password) == 0) {

user_logged_in = 1;

menuState = USER_PAGE;

} else {

lcd_clrscr();

lcd_gotoxy(0, 0);

lcd_puts("Invalid User");

lcd_gotoxy(0, 1);

lcd_puts("Access Denied");

_delay_ms(2000);

menuState = MAIN_MENU;

}

memset(entered_password, 0, sizeof(entered_password));

password_index = 0;

}

}

}

if (bit_is_clear(PINC, BTN_BACK)) {

_delay_ms(50); // Debounce

if (bit_is_clear(PINC, BTN_BACK)) {

if (menuState != MAIN_MENU) {

menuState = MAIN_MENU;

}

}

}

if (bit_is_clear(PINC, BTN_RIGHT)) {

_delay_ms(50); // Debounce

if (bit_is_clear(PINC, BTN_RIGHT)) {

if (menuState == MAIN_MENU) {

menuIndex = (menuIndex + 1) % 5;

} else if (menuState == PASSWORD_MENU) {

if (password_index < 4) {

entered_password[password_index++] = '1'; // Example for demo

entered_password[password_index] = '\0';

}

} else {

switch (menuState) {

case SET_LL: LL++; break;

case SET_L: L++; break;

case SET_H: H++; break;

case SET_HH: HH++; break;

}

}

}

}

if (bit_is_clear(PINC, BTN_LEFT)) {

_delay_ms(50); // Debounce

if (bit_is_clear(PINC, BTN_LEFT)) {

if (menuState != MAIN_MENU) {

switch (menuState) {

case SET_LL: LL--; break;

case SET_L: L--; break;

case SET_H: H--; break;

case SET_HH: HH--; break;

}

}

}

}

}

// Timer0 Overflow Interrupt

ISR(TIMER0_OVF_vect) {

static uint16_t count = 0;

count++;

if (count >= 10) {  // 10 * 1ms = 10ms

count = 0;

timer_flag = 1;

}

}

void timer0_init() {

TCCR0A = 0x00;  // Normal mode

TCCR0B = (1<<CS01) | (1<<CS00); // Prescaler 64

TIMSK0 = (1<<TOIE0); // Enable overflow interrupt

TCNT0 = 0; // Initial value

}

void pwm_init() {

// Set Fast PWM mode with non-inverted output

TCCR1A = (1<<WGM10) | (1<<COM1A1) | (1<<COM1B1);

TCCR1B = (1<<WGM12) | (1<<CS11); // Prescaler 8

DDRB |= (1<<MOTOR1_PWM_PIN) | (1<<MOTOR2_PWM_PIN); // Set PWM pins as output

}

int main(void) {

// Initialize ADC, LCD, Timer, PWM and I/O

ADC_init();

LCD_init();

pwm_init();

timer0_init();

DDRD = 0xFF; // Set LED pins as output

DDRC = 0x00; // Set button pins as input

sei(); // Enable global interrupts

while(1) {

if (timer_flag) {

timer_flag = 0;

current_temp = ADC_read(LM35_PIN);

current_ldr = ADC_read(LDR_PIN);

update_LCD();

control_motors();

update_LEDs();

handle_buttons();

}

}

}